Kalpani River SFINCS Flood Model — Speaking Notes (3–5 min)

Why this matters

- This is a flood simulation model for the Kalpani River watershed (Mardan/Nowshera) — essentially a digital twin of how the catchment behaves in rainfall.

- That's the core value for flood preparedness: you get depth and extent per scenario, from an ordinary rainy day up to a 100-year storm, before the water shows up.

- For urban planning: it flags exactly which buildings, roads, and drainage points are exposed — so decisions like culvert sizing, embankment height, or where not to build can be based on this watershed's real behavior, not generic assumptions.

What was done and how

- Built the model in SFINCS/HydroMT, using FABDEM — a bare-earth terrain dataset

- Enforced a strict rule: no assumed rainfall numbers. Every scenario is either a real historic event (2006, 2010 floods) or computed from real satellite/station rainfall records (CHIRPS, GPM IMERG, PMD).

- Ran a scenario matrix from routine monsoon days up to statistical design storms (5–100 year return periods), plus the two historic events for validation.

- Published an interactive dashboard showing peak impact and flood timeline from these real outputs.

Scenarios incorporated — and why

| Scenario | Rainfall duration | Total sim. duration | Rainfall depth | Data source |
| --- | --- | --- | --- | --- |
| Ordinary monsoon day (P50) | 24 hr | ~4 days (24h rain + 3-day recession) | 7.9 mm — P50 of real Jul–Sep wet-day series | CHIRPS daily, this watershed |
| Notable monsoon day (P75) | 24 hr | ~4 days | 18.3 mm — P75 of same series | CHIRPS daily |
| Moderate / minor-flood threshold (P90) | 24 hr | ~4 days | 33.9 mm — P90 of same series | CHIRPS daily |
| Illustrative ordinary high-rain day | 24 hr | ~4 days | 75.3 mm (25 Jul 2011) — one real dated value from the record | CHIRPS daily |
| 5 Aug 2006 (Kalpani bridge collapse) | 3-day window (3–5 Aug 2006) | ~4 days | 81.8 mm (CHIRPS basin-mean for this window; no independently published mm figure — event is validated on discharge instead, 1,951/2,285 m³/s, Ullah et al. 2016) | CHIRPS daily, real dates |
| Jul 2010 (nationwide monsoon flood) | 4-day window (27–30 Jul 2010) | ~4 days | 202.4 mm (CHIRPS basin-mean for this window; validated on discharge, 1,901/3,361 m³/s, Ullah et al. 2016) | CHIRPS daily, real dates |
| Design storms, T = 5/10/25/50/100 yr | 24 hr total, shaped with one consistent hyetograph (SCS Type-II or alternating-block) | ~4 days | T5 82 / T10 95 / T25 112 / T50 124 / T100 137 mm — Gumbel/GEV fit to 44-yr real CHIRPS annual maxima (PMD station data cross-check where available) | CHIRPS 1981–present + PMD |

{

- Every scenario is run for the same ~4-day simulation window: 24 hours of rainfall input plus a 3-day recession tail, so the model has time to drain and the true peak isn't cut off mid-event.

- CHIRPS (daily, satellite+gauge, 1981–present) drives the daily-total-based scenarios: the climatology percentiles, the two historic events, and the design-storm statistics.

- GPM IMERG (half-hourly, 2000–present) is used only where sub-daily timing matters — deriving the 1–3 hr burst that shapes the design storms, since a flashy hill-front catchment like Kalpani responds to short bursts, not just the 24-hr total.

- Rainfall depth values for the climatology, historic-event, and design-storm rows are computed by running the CHIRPS/IMERG pull against the model's final watershed polygon — that's a deliberate part of the method (no assumed numbers), so the specific mm figures in the table above come from that pull rather than being fixed in the plan itself.

}

Other inputs into the model

- Terrain (DEM): FABDEM — bare-earth elevation grid, defines the ground surface the water flows over.

- Watershed boundary: delineated from the DEM, defines the model domain and confirms the single closed outlet.

- Land cover: ESA WorldCover 10m — used for surface roughness (Manning's n) and as an input to runoff (Curve Number) estimation.

- Soil texture: SoilGrids 250m (ISRIC) — clay content, used to derive Hydrologic Soil Group for runoff behavior.

- Building footprints

Future incorporations

- No building type/occupancy/construction-material data anywhere in the pipeline — only geometry, height, depth, vulnerability class. This is the same national-level gap NDMA's Infrastructure Advisory & Project Development (IA&PD) wing has flagged for their own Infrastructure Risk Atlas — a real alignment opportunity, not just a missing field.

Ready to build now (no new data needed)

- Hazard rating / vulnerability zone polygons — standard depth & velocity formula (HR = depth × (velocity+0.5) + debris factor), turning per-pixel/per-building hazard into zone polygons planners can point to.

- A hypothetical sample building (e.g. a warehouse) with placed assets, animated against each scenario's water level — makes the tool tangible without needing real building-composition data.